SAFETY DATA SHEET

Vincit DS

FLUTRIAFOL 2.5% w/w DS

Table of Contents:

| 1. | Identification of the substance/preparation |  | 9. | Physical and chemical properties |
| --- | --- | --- | --- | --- |
|  | and of the company/undertaking |  | 10. | Stability and reactivity |
| 2. | Composition/information on ingredients |  | 11. | Toxicological information |
| 3. | Hazards identification |  | 12. | Ecological information |
| 4. | First aid measures |  | 13. | Disposal considerations |
| 5. | Fire-fighting measures |  | 14. | Transport information |
| 6. | Accidental release measures |  | 15. | Regulatory information |
| 7. | Handling and storage |  | 16. | Other information |
| 8. | Exposure controls/personal protection |  |  |  |

| 1. | IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING |
| --- | --- |

|  | Product Name | FLUTRIAFOL 2.5% w/w SC |
| --- | --- | --- |
|  | Supplier Emergency Phone No. | CHEMINOVA A/S P.O. Box 9 DK-7620 Lemvig Denmark +45 97 83 53 53 |

| 2. | COMPOSITION/INFORMATION ON INGREDIENTS |
| --- | --- |

| 2.1. | ACTIVE INGREDIENT: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CAS Name | 1H-1,2,4-Triazole-1-ethanol, -(2-fluorophenyl)--(4-fluorophenyl)- |  |  |  |
|  | CAS No. | 76674-21-0 |  |  |  |
|  | IUPAC Name | (RS)-2,4-Difluoro--(1H-1,2,4-triazol-1-ylmethyl)benzhydryl alcohol |  |  |  |
|  | ISO Name | Flutriafol |  |  |  |
|  | EC No. (EINECS No.) | None |  |  |  |
|  | EU Index No. | – |  |  |  |
|  | EU Classification | Xn;R20/21/22-48 R52/53: Harmful by inhalation, in contact with skin and if swallowed. Danger of serious damage to health by prolonged exposure. Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |  |  |  |
|  | Empirical Formula | C16H13F2N3O |  |  |  |
|  | Molecular Weight | 301.29 |  |  |  |
|  | Structural Formula |  |  |  |  |
| 2.2. | TYPICAL CONTENT: |  |  |  |  |
|  | Active Ingredient |  | Flutriafol | 2.5 | % by weight |
|  | Other Ingredients |  | Emulsifier, etc. | 8 | % by weight |
|  |  |  | Carrier | to 100 | % by weight |

| 2.3. | Material Use | Fungicide |
| --- | --- | --- |

| 2.4. | EU Classification of the Product | None |
| --- | --- | --- |
| 2.5. | WHO Classification | None (Technical product unlikely to present acute hazard in normal use) |

| 3. | HAZARDS IDENTIFICATION |
| --- | --- |

| 3.1. | Health Hazards (Acute and Chronic) | The active ingredient flutriafol is harmful by ingestion, by skin contact and by inhalation and may cause liver damage on chronic exposure. The product can cause slight eye irritation. |
| --- | --- | --- |
| 3.2. | Signs and Symptoms of Exposure | To our knowledge, adverse effects in humans have not been reported. There were no observable symptoms i animal tests. |
| 3.3. | Environmental Hazards | See 12. |

| 4. | FIRST AID MEASURES |
| --- | --- |

| 4.1. | Emergency and First Aid Procedures | If in eyes: Immediately flush eyes with large amounts of water or eyewash solution, occasionally opening eyelids, until no evidence of chemical remains. Get medical attention if irritation persists. |
| --- | --- | --- |
|  |  | If on skin: Immediately remove contaminated clothing and shoes and flush skin with much water, followed by washing with water and soap. Get medical attention if irritation develops. |
|  |  | If swallowed: Inducing vomiting is not recommended. Get medical attention. |
|  |  | If inhaled: If experiencing any discomfort, remove the exposed person to fresh air and obtain medical advice. |
| 4.2. | Note to Physician | There is no specific antidote for exposure to this material. Treatment of exposure should be directed at the control of symptoms and the clinical condition. Administration of activated charcoal or gastric lavage can be considered. If the exposed person exhibits signs of severe poisoning, concurrent absorption of additional toxins should be considered. |

| 5. | FIRE-FIGHTING MEASURES |
| --- | --- |

| 5.1. | Extinguishing Media and Procedure | Dry chemical or carbon dioxide for small fires, water spray or foam for large fires. |
| --- | --- | --- |
|  |  | Use water spray to keep fire-exposed containers cool. Approach fire from upwind to avoid hazardous vapours and toxic decomposition products. Fight fire from protected location or maximum possible distance. Avoid heavy hose streams. Dike area to prevent water runoff. Firemen should wear self-contained breathing apparatus and protective clothing. |
| 5.2. | Hazardous Decomposition or Byproducts in a Fire | The essential breakdown products are: hydrogen fluoride, nitrogen oxides, carbon monoxide, carbon dioxide and various fluorinated organic compounds. |
| 5.3. | Unusual Fire and Explosion Hazards | See 10.1. |

| 6. | ACCIDENTAL RELEASE MEASURES |
| --- | --- |

| 6.1. | Personal Protection | Observe all protection and safety precautions when cleaning up spills (see 8). |
| --- | --- | --- |
| 6.2. | Steps to Be Taken in Case of Spill | Stop the source of the spill immediately if safe to do so. Contain the spill to prevent any further contamination of surface, soil or water. Reduce and avoid formation of airborne dust as much as possible. |
|  |  | Spills on the floor or other impervious surface should be swept up immediately and placed in suitable containers. Rinse area with strong industrial detergent. Absorb wash liquid with suitable inert absorbent such as Fuller’s earth, bentonite or other absorbent clay and place contaminated absorbent in the same containers. |
|  |  | Spills in water should be contained as much as possible by isolation of the contaminated water. The contaminated water must be collected and removed for treatment or disposal. |
|  |  | Spills that soak into the ground should be dug up and placed in suitable containers. |
|  |  | The used containers should be marked. Any collected spill material, contaminated waste or absorbent must be disposed of in accordance with the instructions provided under Disposal (see 13). |
|  |  | Washings must be prevented from entering surface water drains. Uncontrolled discharge into water courses must be alerted to the appropriate regulatory body. |
|  |  | The product may be disposed of through proper incineration. |

| 7. | HANDLING AND STORAGE |
| --- | --- |

| 7.1. | Precautions to Be Taken in Handling | See Personal Protection, Section 8. |
| --- | --- | --- |
| 7.2. | Precautions to Be Taken in Storing | Storage at temperatures not exceeding 25°C is recommended. Keep container tightly closed. Protect from frost. |
|  |  | Do not contaminate water, foodstuffs, feed or seed by storage or disposal. |
| 7.3. | Fire and Explosion Precautions | Protect from exposure to fire and heat. |

| 8. | EXPOSURE CONTROLS/PERSONAL PROTECTION |
| --- | --- |

| 8.1. | Respiratory Protection | – |
| --- | --- | --- |
|  | Protective Gloves | Wear heavy duty natural rubber gloves. |
|  | Eye Protection | Wear safety glasses. |
|  | Other Protection | Wear coveralls or long sleeved shirt and long pants. Wear shoes plus socks. |
| 8.2. | Work/Hygienic Practices | If handled indoors, provide mechanical exhaust ventilation. |
|  |  | Persons working with this material for a longer period should be careful to minimise exposure. See 11.5. |
|  |  | Keep all unprotected persons and children away from working area. |
|  |  | Before removing gloves, wash them with water and soap. Always wash hands, face and arms with water and soap before eating, drinking or smoking. |
|  |  | After work, take off all work clothes and shoes. Shower, using water and soap. Wear only clean clothes when leaving job. Do not wear contaminated clothing. Wash protective clothing and protective equipment with water and soap after each use. |

| 9. | PHYSICAL AND CHEMICAL PROPERTIES |
| --- | --- |

| 9.1. | Physical State | Solid (powder) |
| --- | --- | --- |
| 9.2. | Colour | Red |
| 9.3. | Odour | – |
| 9.4. | Melting Point | Not determined |
| 9.5. | Boiling Point | Not applicable |
| 9.6. | Specific Gravity | 0,816 g/cm3 |
| 9.7. | Vapour Pressure | Flutriafol: 5.3 x 10-11 mm Hg at 20°C |
| 9.8. | Viscosity | – |
| 9.9. | Solubility in Water | The product can be dispersed in water. Flutriafol: 130 mg/l at 20°C |
| 9.10. | Solubility in Organic Solvents | Soluble in acetone, dichloromethane |
| 9.11. | Partition Coefficient n-Octanol/Water | Flutriafol: Log Kow = 2.29 |
| 9.12. | pH | 8.45 (5% aqueous suspension) |
| 9.13. | Flash Point | Not flammable |
| 9.14. | Autoignition Temperature | Not available |
| 9.15. | Flammable Limits | Not available |

| 10. | STABILITY AND REACTIVITY |
| --- | --- |

| 10.1. | Thermal Decomposition | Stable at ambient temperatures. |
| --- | --- | --- |
| 10.2. | Hazardous Decomposition or Byproducts | See 5.2. |
| 10.3. | Materials to Avoid | – |

| 11. | TOXICOLOGICAL INFORMATION |
| --- | --- |

| 11.1. | Health Hazards |  | See 3.1. |  |  |
| --- | --- | --- | --- | --- | --- |
| 11.2. | Route(s) of Entry | - Ingestion | LD50, oral, rat: > 2000 mg/kg*) |  |  |
|  |  | - Skin | LD50, dermal, rat: > 2000 mg/kg*) |  |  |
|  |  | - Inhalation | LC50, inhalation, rat: – |  |  |
|  |  |  | *) Estimate based on measurements on a similar product |  |  |
| 11.3. | Irritancy |  | Slightly irritating to skin and eyes. |  |  |
| 11.4. | Allergic Sensitization |  | Not a skin sensitizer. |  |  |
| 11.5. | Chronic Toxicity |  | Repeated exposure may cause liver damage even at low exposure levels. The Lowest Observed Effect Level for this effect has been found to be approx. 15 mg flutriafol/kg bw/day in a 90-day feeding study in rats. |  |  |
| 11.6. | Carcinogenicity |  | Flutriafol is not carcinogenic. |  |  |
| 11.7. | Effects on Reproductivity |  | No effects on reproductivity are found for flutriafol at maternal non-toxic doses (10 mg flutriafol/kg bw/day). |  |  |
| 11.8. | Teratogenicity |  | No teratogenic effects are found. |  |  |
| 11.9. | Mutagenicity |  | Flutriafol is not mutagenic. |  |  |

| 12. | ECOLOGICAL INFORMATION |
| --- | --- |

|  | Flutriafol has moderate mobility in soil. Absorption depends on soil pH and organic matter content. Flutriafol is not readily degradable. The product is harmful to many aquatic organisms. The active ingredient flutriafol has a low potential for bioaccumulation. The acute toxicity of flutriafol to wildlife species is: |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Fish | 96 h-LC50, rainbow trout (Salmo gairdneri) | 61 mg/l |  |
|  | - Invertebrates | 48 h-EC50, daphnids (Daphnia magna) | > 78 mg/l |  |
|  | - Birds | LD50, mallard duck | > 5000 mg/kg |  |
|  | - Bees | LD50, honey bee (Apis melliflora) | > 2 g/bee |  |

| 13. | DISPOSAL CONSIDERATIONS |
| --- | --- |

| 13.1. | Waste Disposal Method | Spill and waste disposal procedures in accordance with state and local regulations must be followed. |
| --- | --- | --- |
|  |  | Do not contaminate water, foodstuffs, feed or seed by storage or disposal. |
| 13.2. | Container Disposal | Triple rinse (or equivalent). Then offer for recycling or reconditioning, or puncture and dispose of in a sanitary landfill. However, if other procedures are prescribed by state or local regulations, these must be followed. |

| 14. | TRANSPORT INFORMATION |
| --- | --- |

|  | UN Classification | Not classified as hazardous material for transport |
| --- | --- | --- |

| 15. | REGULATORY INFORMATION |
| --- | --- |

| 15.1. | IN THE EU: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Classification and Labelling (according to 1999/45/EC as amended): | None |  |  |  |

| 15.2. | THRESHOLD LIMIT VALUES: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Year |  |  |
|  | Flutriafol |  | 2000 |  | TLV not established. A TLV-TWA of 1.5 mg/m3 is recommended by the manufacturer. |
|  | Amorphous silica | OSHA (USA) PEL | 2000 |  | 20 mppcf or 80 mg/m3 % SiO2 |
|  |  | ACGIH (USA) TLV | 2001 |  | TWA 10 mg/m3 |
|  |  | NIOSH (USA) REL | 2000 |  | TWA 6 mg/m3 |
|  |  | Germany, MAK | 2001 |  | TWA 4 mg/m3, inhalable dust fraction |

|  |  | However, other threshold limit values defined by local regulations may exist and must be observed. |
| --- | --- | --- |

| 16. | OTHER INFORMATION |
| --- | --- |

|  | The information provided in this Safety data sheet is believed to be accurate and reliable, but uses of the product vary and situations unforeseen by Cheminova A/S may exist. Regulatory information is subject to changes. It may be necessary for the user to check the validity of the information under local circumstances. |
| --- | --- |